GF2LD:    FBC/GE/GF loader/debugger

12/11/84			User's Guide

================================================================

LATEST  CHANGES

m	toggle monitor mode

ia	initialize and abort test (e.g. after pipe hung)

gt0-b	new test vectors

Gv	depth-cued vectors - use after gt9

GD	depth-cued dots - use after gt9

T	GE token port test

ir	initializes scratch tables

================================================================

This program is a 68000-based tool for FBC-based debugging of IRIS

hardware and microcode.  To run it, download one of the following:

gfld			( GF1 version -- in boot directory )

gfpm2			( GF1 version for workstation )

gf2ld			( GF2 version for PM1)

gf2pm2			( GF2 version for PM2)

The program is an interactive command interpreter with some built-in

facilities for testing the Geometry Engine pipeline and FBC operation.  It

compiles two different ways, for use with wirewrap GE and FBC boards or with

GF1 boards.  It has a nested keyboard macro feature for encapsulating and

repeating command sequences.

STRING NOTATION

In this document, brackets are used as syntactical tools:

<word>	Some substring referred to in the text as "word" or as word.

"word" may be one or several characters.

[x]	Character 'x' is optional.

[x,y]	Either the character 'x' or 'y'.

COMMANDS

Each command line starts with a one-character command ("cmd").  This by

itself may invoke a test sequence.  It may also be followed by one or more

modifiers consisting of single characters or numeric values.  For example,

print and store commands are followed by a one-letter operand ("which"),

followed optionally by a one- letter mode "how".  "How", which usually

designates verbose text ('t') or not-verbose ('n'), defaults to 'n'.

The general form of a print/store command line is:

<cmd>[<which>][<how>][<field>][<sep>][<val>]

Each field option represents one typed (or stored) letter in a command line.

COMMAND SUMMARY

?	help -- print command list

b	bpc command

B	breakpoint set

c	call macro

C	macro diagnostic

d	dismiss program interrupt

f	data file and FIFO operations

g	geometry system command

G	geometry system: special test vector

i	initialize ge/fbc hardware

I	initialize scratch tables

m	toggle monitor mode

M	force GE mask word

o	open microcode location

p	print

q	quit current command level

Q	quick GE confidence test

r	ram read/write tests

s	store

S	setup for GE tests

t	canned board test procedure

T	GE port token flag test

U	unset breakpoint(s)

v	user test vector definition

V	force microcode version no.

w	write microcode buffer to hardware

W	write microcode, preserving breakpoints

<CR>	print next microcode location

^R	(control-r) - re-init screen if iris screen is default

meta-commands:

/	start [multi-line] macro definition

/<char> end macro definition; macro is named <char>

WHICH summary ( for p and s commands )

c	DI bus, fbccode field (4 bits) (GF1)

f	status flag register

i	DI bus (16 bits)

m	microcode memory

b	block of microcode

o	output register

p	pixel data (GF1)

General Remarks

Note that command symbols are case-sensitive; there are only a few

upper-case commands.

A '?' can be typed in place of or after a command letter in most

cases in order to obtain help on the command or option.

Some actions may cause an FBC or GE interrupt to be generated.  If

this interrupt is not expected by the test code, a special command

mode is entered.  See below, INTERRUPT COMMANDS.

Note (GF2) that microcode is built up in a local 1Kx4 wd. buffer;

contents are transfered to the alpha hardware using "w".

A working version of microcode is brought into the buffer

along with the program.

Macro calls can be nested to 32 levels.  It is best if macro calls

in general do not cause interrupts or require typed input.

White space is ignored in directly typed input, but should be absent

in macro bodies.

Numeric fields are hex, with lower-case letters, terminated by any

non-white non-numeric or by <CR>.

TESTS

t

For a quick go/no-go test of the GF1 hardware, just type t<CR> --

This will attempt to initalize the FBC and GE system, test the

scratch ram several times, draw some test pictures, and exercise

the geometry pipeline.  It executes approximately the equivalent of:

sffb  si<n>  pi	(16 tests) (GF2)

rm		(GF2)

w		(GF2)

is

rs  (5 times)

bt

if

gm0

gm2

gm1

T		(GF2)

cs

cf

cP  (with extra checks)

cp

A number of special error messages may appear.  Their meanings are

as follows:

reset error

The FBC didn't reach state 40 after initialization.

no frame buffer

The acknowledge line from the Update Control was found high

or floating.

GE(s) missing:

With the GE reset active, some GE's are trapping.

GEREQFBC not clear

Spurious request out of the tail FIFO to the FBC.

pipeline or FBC inrjust error -

means that a word of data sent down the pipe did not

appear or was not readable in the FBC's "RJUST" input reg.

Prints the bad data.

FBC inljust error

data reached the FBC but the value in the "LJUST" register

was wrong.

not enough GE's -- skipping test

the test vector requires (10 or) 12 GE's to run.

feedback error: ...

the FBC feeds vector results back twice;

the two reads differed. (GF1)

Error at word <w>

not all of the GE test vector could be sent.

no feedback from GE test

no interrupt occurred to feed test data back to host.

Error on wd. <w>: <d> should be <v> (q to quit)

the GE output was wrong at word <w> in the sequence.

Other hardware test commands include:

Q		quick pipe confidence test for WS w/o tty

i		basic FBC functionality

rs		scratch ram test (e.g. is  rs)

rm		microcode ram test (GF2)

T		GE port/token test

gt<num>		picture-drawing test (e.g.  ir  gC  gt0-6)

gm<num>		multibus-based drawing (e.g.  if  gm0 gm1 gm2)

gm0 clears the screen.

gm1 draws a white circle.

gm2 draws 8 dashed vectors.

Gb		transformed boxes (e.g. ii  gC  Gb)

T

Token test.

Once GE pipe shown to work, can run token test.  Prints errors:

no initial clear	Token active without having been sent.

unprovoked interrupt	Token interrupt enabled and a spurious one

came, before or after a word sent.

no interrupt		After a token sent, no interrupt occurred.

TOKEN_BIT_ doesn't go active	After token sent, flag bit not active.

can't clear		Token flag bit can't be cleared.

Also, see below under TEST SEQUENCES.

INIT

i[<mode>]

Before doing any operations that use the FBC or GE pipe, perform

an "i".  This will set up everything on the GF board to a clean

state.  It does not assume that there is any BPC hardware in the

system.  The number (GEfound) and arrangement (GEmask) of GE's

on the board is reported, followed by the scratch size and microcode

version.

The microcode must be loaded via the "w" command before any init

command (GF2).  The w is needed only after power-up or microstore

testing.  If an init command is done befor the w, do a "V" command

to reset the microcode version number.

Depending on the hardware present in the system, the flag registers

can be set up in an optimum way by specifying the mode explicitly:

r run mode

For when all hardware is available and useable.  Runs findge()

to determine which GE chips are present in system, and reports

results.  Initializes color map. Also performs an 'I' to

initalize scratch tables.

f  frame buffer mode

If the bit plane hardware is present and useable, but the GE system

is not.  Like 'r' reset, but doesn't try sending to GE pipe.  Inits

color map.

b  bpc command mode

For when testing of individual BPC commands is desired (the 'b'

commands)

s  standalone mode

For when no GE system or BPC is present or useable. Will not attempt

to initialize color map.

d dummy pipe mode

For when dummy wafers are installed as FIFOs, but all other hardware

is useable.  Does not run findge().  Useful with SD, gP, and gd

commands.

i interrupt mode

Enables FIFO high-water interrupts.  For running G drawing tests,

ga tests, and gB tests.  Initializes color map on DC board.

I interrupt standalone mode

Same as i but assumes no BPC hardware.

a abort testing

If some test causes the pipe to hang (with HIWATER stuck high),

a new command shell will be entered.  Type ia before exiting this

level in order to attempt to unhang the test.

t tests library reset procedure.

I

initializes any scratch ram tables used by the microcode.  Microcode

must be written on the board and FBC must be in a command input

state.  Performed automatically by "ir".

WRITE

w  W

To use the ram-microcoded FBC or GF2, a "w" must be performed at

least once after system power-up.  This transfers the current compiled-in

microcode to the microcode ram.  The contents are verified after writing

(GF2).  The W command preserves any breakpoints set by the user.

PRINT

In successive print commands, "which" defaults to the previous

command.

*)	p[m][<how>][<addr>]

Prints microcode buffer contents, address <addr>.

"How" can be:

n 	numeric value of state divided into 16-bit chunks, i.e

40	3cc0	e5	30

= bits:		0-15	16-31	32-47	48-55

= field:	 0	 1	 2	 3

The field designation (0..3) is used in the numeric

STORE command below.

t	text. Prints each micro field in symbolic notation.

The fields are represented by capitalized 3-letter

designations:

ALU	2903 opcode, alu part

BRA	branch address (DI field)

CDN	condition code

CIN	carry-in

CLK	clock period

DST	2903 opcode, destination part

FBC	FBCCODE field: onboard/output commands

HIB	"HIGHBYTE" signal for alu output ctrl;

"SWAPBYTE" on GF2

IOC	I/O operation

MXA	alu A port selection

MXB	alu B port selection

RAM	ram operation

RGA	alu A port register no.

RGB	alu B port register no.

RJU	right-justified input reg selection;

carryin/clock control on GF2

SEQ	sequencer opcode

<CR>	previous mode

"Addr" can be:

<hex number>	the desired address

.		the "current addr" pointer - the previously

.+ <hex no.>	printed or stored location.  Relative address-

.- <hex no.>	ing from '.' can be used, e.g. '.-3f'.

,		equivalent to '.+1'

NOTE: 't' must be explicit (preceding) if

desired

<CR>		defaults to 0

In all cases of addr, the "current addr" ('.') is set to

whatever address is calculated.

SPECIAL: if previous cmd/which was pm, and just a <CR> is entered,

the next <addr> in sequence will be printed.

*)	p[f][t]

reads and prints flag register. Optional verbose text mode prints

the meaning of each bit.

*)	p[c]	(GF1 only)

prints DI bus bits corresponding to the fbc code register

output.  The code is enabled onto the bus by setting the

flag register to the proper value using "sf".

*)	p[i]

reads and prints DI bus status.  Useful after

setting flags to spy on input or output registers.

*)	p[o]

reads and prints FBC output register.

*)	p[p]	(GF1 only)

prints value of pixel data currently present on bit plane boards.

OPEN

*)	o[m]<addr>

Works like p[m]<addr> but nothing is printed. Useful for macro

bodies for patching, etc.

STORE

In successive store commands, "which" defaults to the previous

command input.

*)	s[m][t][<field>][<sep>][<val>]

stores into a designated field of the PREVIOUSLY PRINTED OR OPENED

microcode address.

The field is specified either as one of four 12-bit words

("how" = 'n' or omitted) or as a particular field ("how" = 't').

how = numeric

<field> is 0,1,2, or 3 (see PRINT command)

<sep> is any non-numeric non-white char.

<val> is a hex number.

how = text

<field> is '?' to print out a list of the choices,

or a 3-letter field mnemonic.

<sep> is omitted, or white space.

<val> is a hex value appropriate to the field

or '?' to print a list of possible values and

their meanings, after which the desired <val>

is re-prompted.

*)	s[b][t][<field>][<sep>][<val>][<low>][<high>]

stores into a designated field of all microcode addresses from <low>

to <high>. Otherwise, same as [s][m]....

*)	s[f][<val>]

store flag register with val (defaults to 0).

If sf? is typed, a description of the writeable flags appears.

*)	s[i][<val>]

store input register with <val>. Use in flag-blocked mode to input

commands and data to the FBC.

FILE/FIFO

fo[<flag>][<sep><ignorewords>]

This causes any errors collected as a result of geometry system

testing (e.g. ge1, gp) to be reported, and the error

file to be reset.  If <flag> is zero or omitted, only the total no.

of errors is printed; otherwise if there are any errors the user may

type a character to see each error.  Ignorewords, which defaults to

0, is used to specify the number of expected test vector items to

ignore before starting to compare expected and received values.

fb[<flag>]

Similar to "fo" to be used after gB tests.  Ignorewords is not

specified.

ft[<silent>]

Tests all basic functions of FIFOs with installed GE's configured

as passers.

Unless silent > 0, prints word number after which pipeline fills.

This should be 152 for a 12-GE system, and 2 less for each missing GE.

On GF2, shd be 149 for a 12-GE system, and 2 less for each missing GE.

Used by "cf".

cs  ft1  fo	runs a FIFO test and supresses fill printout

fq

After using the "gp" or similar testing commands, an "fq" may need

to be performed to allow the normal handling of interrupts to

be resumed.

GE

Commands relating to the Geometry Engine section of the system.

GE commands range from very simple flag manipulation to very complex

test sequences.

*)	g<which>[<num>][<sep><num2>]

"Which" can be:

?	help -- print list of commands and size of

GE test vectors (no. of words expected)

f[t]	print flag reg [with text breakdown of bits]

s<num>	store flag reg with <num>

s?	gives breakdown of bits.

d<num>	send <num> down the geometry pipeline

E<num>	send <num> down with token bit set.

p	run pipeline complete functional test. Follow w/ "fo".

P	run pipeline PassThru test

Send a test vector for all 10 or 12 GE's in the pipe

(as determined by "ir" or "S").  Use "fo" to retrieve

results.  Before each word is sent, the high water

flag is tested.  The gw command affects the delay

between words (GF1).

These tests are best called via "S" / "c[p,P]".

t<num>	run test data file <num> thru pipeline.

The tests for GF2 are:

0	setup and clear screen

1	set up 4 viewports, fat lines

2	8 colored vectors

3	block clear, characters

4	cursor, more characters

5	circular polygon

6	outline polygon

7	block fill, pixel copy

8	pixel read/write

9	4 depth-cued vectors

a	hitmode test

m<num>  run alternate data file <num> thru FBC only, via

Multibus.

0	Setup, clear screen

1	circle polygon

2	vectors

3	text (tops chopped off)

wr

w<num>  specify inter-word wait for 'm' tests.	(GF2)

When gfld is first loaded, wait is initialized

to 400 (slow).  Use <num> of 1 for fast sending when

'gt' and 'gv' are used.  Set to at least 400 for 't'

and 'gm' tests.  set to 'r' for random wait times

when looking for noise problems.

i	hard-reset geometry pipeline only.

C<low>[<sep><high>]

configure pipeline with chip nos. <low> to <high>

active and the rest passing data.  If low is 0, all

chips are active.  If high is omitted it is set to low.

"gCa" isolates chip 10 (last clipper on GF1).

"gC1-4" isolates the four matrix multipliers.

NOTE: gC only configures chips that were detected when

"ir" was called.

"gCd" initializes all installed chips to be passers.

T	set GE test mode.  When gfld is first loaded, or

after typing 'gT[0]', 'gC' works as described above

to test 1 or more GE's; after saying 'gT<n>',

GE number <n> can be configured as chip <num> by

invoking gC<num>.  This allows any one GE in the pipe

to be tested as any chip type.  In this mode the

line prompt is 'T'.  Error message printed if chip

<n> is not installed.

D	for debugging, prints out the sequence of config words

sent to the GE's when the gC command is typed.  Note

however that words for all 12 GE's, whether installed

or not, are listed (GF1).  Mode toggles.  Also prints

words sent by gefind(), other routines (GF2).

v<n>	sends the contents of the user test vector down

the pipe n times.  The user test vector is set up

by the 'v' commands (see).

l	continuous pipeline test: repeatedly sends the

user test vector down the pipe (GF2).

e<num>[<sep><num>]	('e' not present in GF1 version)

perform a test on isolated geometry engines.

After initiating the DBLFEED microcode ("S")

and executing "gC1", doing a "g1" will perform a test

on the first matrix multiplier. (See Appendix.)

"ga" tests chip 10 (scaler).

The numbers given here and the

numbers used with gC should always match.  However,

gT<n> may be called to test a particular chip in

the pipe as a non-standard type.

B<low>[<sep><high>[<sep><prefixes>]]

This works like a combination of gC<low><sep><high>

and g<low><sep><high> but pre-loads the pipe, which

has been blocked at the FBC input, with <prefixes>

no-ops.  Then the block is opened after the pipe has

been filled to the high water mark with the selected

test vbector; then the rest of the test vector is sent

at high speed.  This provides a pipe test in which

the GE's never have to wait for input.  Use directly

after an S command (no gC needed).

a[<which>][<section>][0,1]	(GF2)

Geometry Accelerator tests.  Set up and run test

vectors for GA's.

ga[0..f]	send down one of the test vectors.

Perform the appropriate pipe config-

uration sequence for testing either

head or tail GA, then send the test.

Follow with "fb" to retrieve results

and set up for next test.  Precede

with "SI" the first time.

ga[h,t][c,f][0,1]

defeat findge()'s idea of whether

GA's are installed.  Enable or

disable a section of one of the GA's:

gahc0	disable head cfp

gatf1	enable tail fifo

...	etc.

Use this after each "SI" command

as needed.

RAM

rs	Perform test of scratch ram.  Requires working ram test

microcode.  Do an "is" first.

rm<num>	Performs test of  microcode rams on a GF2.  Num specifies

the number of locations to test in the random data part of

the test.

rv	Verifies that contents of microcode rams match buffer copy.

Reports any differences one at a time.

rl<dat>	Looping scratch test.  Allows scoping of writing word "dat"

to scratch.  Forever.

rd<n>	Dump scratch.  Displays 16 words of scratch ram starting from

location n.  FBC must be in a command input state.  Prompts

for whether you want to see the following 16 locations.

r2	2903 register display.  Displays (non-destructively) contents

of 2903 registers.  FBC must be in a command input state.

rr<loc>	read microstore location "loc" and print contents.  The

location format is "saaa" where s is the slice number (0..3)

and aaa is the state address within the slice.

rw<loc><sep><dat>  write microstore location loc with data dat.

The loc format is as for "rr".

VECTOR

v<op>[<in/out>]

This set of commands allows construction of test vectors or config-

uration sequences that can be rapidly deployed via the 'gv' command.

A separate input and output vector can be constructed, and the latter

can be used to check pipeline output via 'fo'.

v?	displays a list of 'v' commands, AND initializes the input

vector to all-8's.  There is a max of 100 words

in each vector.

vi<datum>

vo<datum>  set the next word in sequence in the input or output

vector to datum.  The starting point for storing can

be set via 'vd'.

vpi	prints contents of input vector

vpo	ditto output vector

vdi<num>

vdo<num>   These point to the last word in the desired vector.

'vdi0' describes a 0-length input vector, so that the

next 'vi' will specify the first vector word.  'vd'

can be used to move the word pointer for editing vectors.

BPC COMMANDS

These are meaningful only if "ib" is first performed.

bs<regno><sep><data>

Strobes the BPC register regno with given data.

Type "b?" for a listing of register numbers.

bc<cmdno><sep><data>

Issues the BPC command cmdno, supplying the given data.

Type "b?" for a listing of the command codes.

bb<regno>

Reads and prints the contents of buffer regno.

br<cmdno>

Performs command cmdno as a read, and prints the data returned by the

UC4.

bt

Performs a floating-1 test in font ram location 0 as a test of the

interface data path.

INTERRUPT COMMANDS

Interrupts may be caused by microcode instructions, diagnostics,

GE or FIFO signals, or by the onset of vertical retrace.

Many of the test routines like rs, T, etc. generate interrupts

that are automatically intercepted.  When an unexpected

interrupt occurs, the user is given options as to how to

proceed.

continue -- <cr>

Just dismiss the interrupt and continue (with microcode).

command -- 'c'

Enter a command subshell before dismissing the interrupt. Exit

this subshell by typing 'q'.  While in this subshell, interrupts

won't be reported by the program.

If your commands clear the source of the interrupt, you will return

to the outer command level after the 'q' is typed. Otherwise

a new interrupt may be generated.

go -- 'g'	(Obsolete)

Use after a startup interrupt. Does interrupt acknowledge to

generate 1 clock for microsequencer load. Then sets FBC to normal

run mode and dismisses interrupt.

debug -- 'd'

Like "go" -- but sets FBC to blocked-interface run mode.  Can be

used after a startup interrupt (old FBC) or when things are stuck.

print -- 'p'

Prints the value of the output register and fbccode flags.

Interrupt is then dismissed, and microcode execution continued.

Useful for displaying feedback sequences.

quit -- 'q'

On PM1 version, resets hardware and restarts program at the top.

Pn PM2 version, resets hardware and attempts to clean up before

returning to command level.

Use the fq command after doing "cp" type tests to restore the interrupt

flags.

MACROS

c<name>

There are a few predefined macros useful for testing the Geometry

Engines.  The sequence to test a complete, normally configured

Rev. 1 pipeline is:

ir

S

cp (or cP)	( runs a pipeline test. See 'g' commands )

cp100		( run test 256 more times )

Other predefined macros:

cf	runs cP, filling up the pipe.  Test of the FIFOs.

substitute for cP above.

ci	cycles input handshake, e.g. after si<data>.

cO	no-op; time waster.

c<n>	isolated test of GE number 1..c.  Substitute for cP

above.  Depending on state of 'gT' toggle, will

either test each GE individually or test the first

GE as each type:

1	1st MM   (MM1)

2	2nd MM   (MM2)

3	3rd MM   (MM3)

4	4th MM   (MM4)

5	1st Clip (CL0)

6	2nd Clip (CL1)

7	3rd Clip (CL2)

8	4th Clip (CL3)

9	5th Clip (CL4)

a	6th Clip (CL5)

b	1st Scale(SC0)

c	2nd Scale(SC1)

c#	use to read back the results of a GE test vector

command like gp, g3, or gB.  Prints "0 errors in <n>

words" if all received values match the expected

results.  Otherwise successive errors can be viewed

by hitting 'y'.  Hit any other character to quit.

(Same as "fo1")

c$	Same as c$ but simply reports no. of errors and

continues.  (Same as "fo");

cd,ct,cT,cg,cG		see below.

C<num>

If num is 0, prints a list of all the names of the currently defined

macros and an index number for each.  Set num to the index to see

the contents of a particular macro.

BREAKPOINTS

A breakpoint is an FBC interrupt and optional load of the FBC output

register for printout.  If inserted into running code, the interrupt

commands may be used to print the value or deterine control flow.

Any use of the FBC field controlling the MAR and UC commands is

temporarily overridden.

B[p,n]

B sets a breakpoint at the current microcode address.  Set the current

address using an o or p command.  This operation may destroy the

current state of the FBC, so an init command is needed before running

again.  Bn inserts a breakpoint but does not change the FBC output

register.

Bp gives a list of the currently set breakpoints.

U[<which>][<loc>]

U unsets the breakpoint at the current microcode location.

Ua unsets all breakpoints.  It is the equivalent of "w".

Ul<loc> unsets the breakpoint at location loc.

The W command may be used after modifying the microcode when

user wants to keep the same breakpoints set.

DRAWING TESTS

Precede any of these by "ir  gC".  Then any number of gt's or G's may be run

in any order except as noted.

gt0	Initializes for drawing, clears screen to black.

gt1	Sets up 4 screenmasks for subsequent drawing - a rectangle in each

corner of the screen (4 different sizes).

gt2	Draws 8 colored vectors, one stippled.  If gt1 precedes, lines are

fat.

gt3	Clears part of screen and draws "sgi s UUU" mid-screen.

gt4	Adds "UUU" to string and draws IRIS logo cursor.

gt5	Draws circular polygon

gt6	Outlines circular polygon.  If gt1 precedes, lines are fat.

gt7	Clears part of screen to white and draws wedge-shaped strip.

Tests pixel copy.

gt8	Draws multicolored bar. Tests pixel write and read.  UC must not

be busy drawing to work properly.

gt9	Draws depthcued vectors in an M shape.

gta	Tests hit mode.  Do not use gt9 before.  There may be 3 identical

errors reported.  UC must be non-busy to work properly.

gtb	Draws Z-buffered picture of 2 polygons.  Needs some CD planes.

Gb<n>	Draws spiral of red and white squares n times.

Gd<n>	Draws a colored square and an L-shaped pattern of dots n times.

Default n is a few seconds worth.

Gp<n>	Fills the screen with 16-spaced dots in passtru mode n times.

Gv<x>	Draws a ramp of depth-cued vectors.  Precede with gt9.

The value x specifies the width of the bottom part of the pattern.

GD	Draws a ramp of depth-cued dots at the top of the screen.

Gc<x>	Copies a triangular area of the screen to the bottom center of the

screen.  The value x is the x coordinate of the left vertex of the

source area.

TEST SEQUENCES

Here are a few useful command sequences, presented in order of

functional independence.

t

See description of t command above.

S  cP cp ct cg

GE and GA pipe test sequence.  S should accurately report the number

and position of installed custom chips.  cP runs a passthru test.

cp runs a full test vector, checking only the output of the pipe.

ct runs a full test vector, checking one GE configured at a time.

cg runs the full set of GA test vectors.

SI  cT cG

The cT call is a more stringent version of ct.  See gB command.

The cG call is a more stringent version of cg.  See gaF command.

S  gT<n>  ct      or    SI  gT<n>  cT

Example of how to test complete functionality of the <n>th GE

in the pipe, with all other chips configured as passers.

cd

Performs the equivalent of

ir gC gt gt gt1-7 gt1-7 ....

to test the ability of the system to draw rapidly using

passthru commands.  Should work with any combination of GE's and

dummies.

ir  gC  gt0  gt1 ... gt9  Gv  Gc  Gc300  Gc8  Gc308  gt  gta  gtb

ii  gC  gt  Gd  Gb10  Gp10

A recommended test sequence for microcode & drawing behavior.

is  gm  [ gm1  gm2  gm3 ]

Quick way to clear the bitplanes.  Needs no GE's.

Extra tests are drawing tests that use the FBC Multibus port.

[]--[]--[]--[]--[]--[]--[]--[]--[]--[]--[]--[]--[]--[]--[]--[]--[]--[]--[]--[]