The PM2 proms were not designed for diagnostics.  It was

expected that these functions would be added to the proms,

or a set of diagnostic proms would be created at a later date.

Some minimal functions, however, are performed by the proms

to test basic hardware functionality.  Each of these tests, and

each of the phases of the prom boot process itself, is given a

test code which is displayed on the LEDs when the phase is begun.

Later phases of the boot process may be accompanied by screen i/o

if the VERBOSE configuration switch (switch 3 on the back

panel) is set.

On a hard reset, the proms do the following in the early stages:

start refresh. turn off parity.

If the configuration switches are set to the special

DONTTOUCH environment (switches 3-8 set), the proms skip from this point

to the intermediate initialization sequence.

map page zero logical to physical.

perform various hardware tests:

If one of these tests fail, the action of the prom will be

controlled by the verbose switch (switch 3 on back panel). If

this switch is not set, the proms will halt the processor. The

led should display the id of the test which caused the failure. If

the verbose switch is set, the proms will cycle between displaying

the current test code and an error code in the led (forever). In this

case, the test code is displayed for twice as long as the error code.

A.	test the page map	(test code 0(back) F(on board))

The entire page map is tested by writing various patterns

to it and reading them back.  Certain information during this

test is available in the registers (for examination by the ICE):

a0 - contains the address of one word below the address of

the pagemap entry which failed.

d0 - is the error code.  The error occurred on a read (1)

or a write (0).

d1 - is the pattern which was being written.

B.	test the protection map (test code 1(back) E(board))

The entire protection map is tested similarly to the page

map. The registers contain analagous information to test A.

C.	test the context register (test code 2(back) D(board))

Various patterns are written to and read from the context

register.  The registers set similarly to A and B.

intermediate initialization sequence.

This consists of setting up whatever things the proms need to

get at the world.

A.	map for the prom RAM needs, and set up the prom RAM data

area. (test code 7(back) 8 (board))

The prom needs page 0 and page 7f.  Page zero is used for

the vectors and a communications area that the prom uses to

interface to standalone programs and UNIX.  Page 7f is used

for stack space and for the prom globals.

1. clears page 7f.

2. copies the data currently at 0-0x300 to 0x7f000.

3. clears the area from 8 to 0x200.

B.	stuff interrupt and trap vectors, clear bss.

(test code 8(back) 7(board)).

The trap and interrupt vectors are stuffed.  It is possible

that a trap received between this point and the time the duarts

are set will not be handled gracefully.

The prom bss area is cleared.

C.  test code 9(back) 6(board).

The multibus is reset.

D.	test code A(back) 5(board)

Autoconfiguring is done for the terminal type.  The

multibus I/O space is set up, the duarts are initialized, and

an attempt is made to access the iris keyboard.  If the keyboard

responds, the screen is initialized.  If this succeeds, the

terminal is assumed to be a microswitch keyboard on an IRIS

terminal.  If the keyboard was found, but the screen cannot be

initialized, the message 'Screeninit fail' will be written to

the ASCII terminal assumed attached to port 1.

The PM2 prom monitor sign message is printed in this phase.

E. 	test code B (back) 4(board)

Hook for future expansion.

At this point, if the configuration switches are set to the

magic DONTTOUCH environment, the final setup phase is skipped to.

F.	test code C (back) 3 (board)

Memory is sized and cleared.  If VERBOSE is set,

this appears as a line of X/.'s on the screen. A character

is output for each half meg of memory found. '.' infers

no memory present, and 'X' infers memory was found.

G.	test code D (back) 2 (board)

The map is initialized.  All memory is mapped logically

contiguous, multibus memory is mapped, etc.  (This is

the first memory access via the multibus.)  I/O space is

mapped.  The amount of each type of memory mapped and their

locations is written to the screen if the VERBOSE switch is

set.

final setup phase:

A.	test code 0(back) f(board).

parity is enabled.  The host duart is set to the speed

indiated in the configuration switches.

B.	test code 1(back) E(board).

The main prom routine is called.  This initializes the

serial interrupt handler and the prom internal clock.  If

the DONTTOUCH configuration is selected, nothing else is

initialized.  Otherwise, a download buffer is allocated,

and the duarts are flushed.

C. 	test code 2(back) D(board).

If the fields in the communications area are set to do

a UNIX-initiated boot sequence, this is begun.  Else, the

prom action is defined by the configuration switches.  These

are: (switches 4(LSB) - 8(MSB)):

back panel switch

8---4	function

-------------------

00000 - floppy boot

00001 - tape boot

00010 - enter iris monitor

00110 - talk over the serial line (emulate a

v50 terminal.

01000 - net boot

10000 - disk boot

11111 - if VERBOSE (switch 3) is one, this is the

DONTTOUCH environment.

all boots are done from the file defaultboot on the indicated

device (except netboot, in which case the

default file is defined by the host).

Certain other environments are defined which are

not mentioned here.

Undefined environments default to the iris monitor.