D O C U M E N T A T I O N   F O R

M S - D O S   V E R S I O N  2 . 1

G E T T I N G   S T A R T E D

1.	The first thing to do is check these jumpers on your boards.

DO NOT RUN SETUP ON YOUR HARD DISK!!

JUMPERS THAT MIGHT HAVE CHANGED

SLICER BOARD		JB1 ON REV E AND E2 LOOK ON DRAWINGS PAGE 1

CONFIGURE FOR 27128 EPROM

EXPANSION BOARD	J5 JUMPER 3 AND 4  ( INT 0 )

2.	Now remove the EPROMs from your Slicer board  U6 and U7.

Replace the EPROMs with the ones sent, U6 ODD U7 EVEN.

K E E P   T R A C K   O F   P I N   O N E ! ! !

3.	The next thing to do is COPY YOUR DISKS!!!  Make a copy of

the two disks provided by Slicer Computers using the MS-DOS

format and copy commands, or the diskcopy command.  Save these

original disks for future updates.

If you are having trouble getting running go to the end of

this document for TROUBLE SHOOTING tips.

****	Note that both disks are formated for 40 track double sided

double density.

M S - D O S   D I S K   F I L E S

*** SYSTEM DISK ***

DEVCON   ASM    17156   5-29-86   2:33p

DEVCONB  BIN      977   5-29-86   2:49p		InOut terminal driver

DEVCONI  BIN      872   5-29-86   2:49p		Input terminal driver

DEVCONO  BIN      523   5-29-86   2:49p		Output terminal driver

EMM      BIN     6224   7-28-87   2:24p		(LIM) memory manager

HDMSTRBT BIN      512  12-01-85  10:13p		Master Boot Record(Hard Disk)

CHKDSK   COM     6468  10-19-83   7:51p

COMMAND  COM    15957  11-10-83  12:03p

DEBUG    COM    12223  10-19-83   7:52p

DISKCOPY COM     1409  10-19-83   7:51p

EDLIN    COM     8080  10-19-83   7:51p

FORMAT   COM     6659   5-29-86   1:52p

HFORMAT  COM    11878   1-01-80  12:18a		Dos level hard disk format

MORE     COM     4364  10-19-83   7:51p

PRINT    COM     3808   1-29-85   7:46p

RECOVER  COM     2295  10-19-83   7:51p

SETCLK   COM      369   1-01-80  12:01a		Reads Real-Time Clock

SETRTC   COM     1080   1-01-80  12:02a		Sets Real-Time Clock

SETUP    COM     3840   1-11-86   5:16p

SYS      COM      922  10-19-83   7:51p

EMM      DOC      333   7-29-87  10:32a

README   DOC    	        		this document

BACKUP   EXE    28227  12-12-85  11:13p

CHNGDEV  EXE    16344  12-01-85   9:53p		Utility to change device

EXE2BIN  EXE     1649  10-19-83   7:51p

FC       EXE     2585  10-19-83   7:51p

FIND     EXE     6331  10-19-83   7:51p

HDFORM   EXE    21924   9-29-85   5:22p		Hard Disk Format Utility

LINK     EXE    43988  10-16-85   4:00a

PARK     EXE     2124   5-14-87  12:58p		Park Hard Disk Heads

RESTORE  EXE    26307  12-12-85  11:22p

SLOWDOWN EXE      545   6-08-86   4:22p		Slows down floppy disk access

SORT     EXE     1632   1-29-85   7:48p

SPEEDUP  EXE      545   6-08-86   4:20p		Speeds up floppy disk access

TTYOFF   EXE      521   6-05-86   2:29p		Default extended TTY

TTYON    EXE      521   6-05-86   2:26p		Turns on extended TTY

PARPRINT PRN       11   2-26-86   1:05p		Used with CHNGDEV.EXE

CONFIG   SYS       24   1-01-80  12:35a		128K ramdisk setup

RAMDISK  SYS     1098  12-01-85   4:02p		Ramdisk Utility

CURSBLNK.EXE					Turns cursor blinking on

****	S O U R C E   D I S K    ****

BIOS.ASM			DBG.ASM

IBMBIOS.ASM			TABLES.ASM

NEWCALL.ASM			MEMTEST.ASM

OLDCALL.ASM			HDBOOT.ASM

NEWVIDEO.ASM			SOURCE.DOC

NEWKEY.ASM			BIOS.MAK

BOMB.ASM			BIOSADJ.EXE

SPRN.ASM			LOADER3.EXE

RESET.ASM			STEAL210.ASM

BIOSDATA.ASM			LD4.CMD

IBMPRN.ASM

INIT.IN

SU.EXE

INIT.EXE

SETPORT.BAT

A note first the MS-DOS disk contains a config.sys file that

has a ramdisk driver installed.  If you do not want the ramdisk delete

this file.  If you want to change the size from the default 128K that

information will be given later.  As a note to begining MS-DOS users

a config.sys file with files=10 & buffers=20 can speed up your

hard disk quite effectivly.

NOW REBOOT YOUR SYSTEM FROM THE NEW DISKS AND SAVE THE ORIGINALS!!

C O N F I G U R I N G   Y O U R   S Y S T E M

To configure your system first use the program named SETUP on

the MS-DOS disk. With this program you can set the number of floppy

drives in your system and there step rates.  The default for the

printer is serial port B on the slicer board.  The format for data

transfer out this port is also set by the SETUP program.  The speed

of the dos clock is set with this program.  To configure your system

the best possible run all of the necessary menu selections.  When

you run this program you should have a back up of the disk you made.

If you are changing the setup and booting from a hard disk, change

the set up on a floppy disk then reformat your hard disk as a system

disk from your floppy.  If your printer uses hardware handshaking,

run handshak.exe in your autoexec.bat.

F L O P P Y   A N D   H A R D    D I S K   F O R M A T

There are three format programs supplied with MS-DOS for the

Slicer.  FORMAT.COM is the floppy disk formatter.  When FORMAT.COM

is run there are menu selections for the type of disk drive being

used, other than that it is the same as in the manual.  The

recommended formats for 40 track 5 1/4 inch drives is 9 sectors, and

5 sectors for 80 track drives.

Some floppy disk drives were having problems running at the high

speed that we expected them to.  For these drives we added a bit

to the floppy disk read and write commands which causes a 15 ms

head settling time to be added,  This may not be necessary for your

drives and it slows down disk operations by about a third. Also the

floppy disk spin up time delay speed was made programable.  Although

these conditions are set to there maximum delay at boot time it can

be changed once your machine is running from autoexec.bat.  Once you

get booted try running the program SPEEDUP.EXE.  If you get to many

disk errors slow your drive speed back down with the program

SLOWDOWN.EXE.  The way to set the spin up time is through int f8h.

By making this call with bx register equal to zero the head settle

time is no longer used.  The cx register must contain the refferance

to spin up time with ffff equal about .5 seconds.  The ax register

must contain the channel and function 0079h.

HDFORM.EXE and HFORMAT.COM are used to format hard disks.

HDFORM.EXE is used first and does the actual formatting of the disk.

It also writes the master boot record and sets up the partition table

on the disk.  HDFORM.EXE can also be used to change the partition on

the disk, or to tell the operating system to add a second hard disk

to the system.  This can be done even if the disk has already been

formatted.  When formatting the hard disk the disk parameters need to

be entered.  HDFORM.EXE is set up to default to parameters for a

MiniScribe 3412 drive.  If a different drive is used the parameters

can usually be found in YOUR MANUAL.  The step rate options for

the WD1002-SHD Winchester Disk Controller are as follows.

0 - 3 ms

1 - Ti half step

2 - 3 ms

3 - 3 ms

4 - 200 micro seconds

5 - 70  micro seconds

6 - 30  micro seconds

7 - 15  micro seconds

8 - 2 ms

9 - 3 ms

To set up the paratiton on the hard disk, set the begining and length

for each partition.  The final length must be one less than the number

of sectors on the disk.  If a partition is given a length of 0 sectors

it will not be checked for overlap with other partitions so if a

partition is to be left empty give it a length of 0.  The media ID FOR

MS-DOS IS 1.

HFORMAT.COM is used to set up the fats and directory on the MS-DOS

partition of the disk.  After a disk has been formatted or had its DOS

partition changed HFORMAT.COM should be run.  HFORMAT.COM is used

as described in the MS-DOS manual under FORMAT.

To format your hard disk run HDFORM with HDMSTRBT.BIN on the

same disk.  Reboot and run HFORMAT.

A   L I T T L E   A B O U T   T H E   B I O S

Bios version 2 has the ability to reassign standard input and output

devices.  There is a program supplied with the new version of MS-DOS

that does the reassignment in a simple manner.

To understand how this program works one must know a little about

how the bios channels work.  The bios does all input and output

through numbered channels.  These channels are inturn hooked up to

different numbered devices.  The following table shows the defaults.

CHANNEL #               HOOKED TO INPUT	    OUTPUT

DEVICE #		    DEVICE #

______________________________________________________________________

floppy disks : 0-3 :  floppy disks     : 0 : floppy disks     : 0

hard disks   : 4-5 :  hard disks       : 1 : hard disks       : 1

console      : 6   :  serial port 0    : 3 : serial port 0    : 2

con w/pc exp : 6   :  keyboard         : 7 : video            : 6

printer      : 7   :  serial port 1    : 5 : serial port 1    : 4

aux	     : 8   :  serial port 0    : 3 : serial port 0    : 2

memory size  : 9   :  memory size      : D : memory size      : D

port 1       : A   :  serial port 0    : 3 : serial port 0    : 2

port 2       : B   :  serial port 1    : 5 : serial port 1    : 4

port 3       : C   :  expansion port 0 : 9 : expansion port 0 : 8

port 4       : D   :  expansion port 1 : B : expansion port 1 : A

pc board     : 10  :  keyboard         : 7 : video            : 6

centronics   : 11  :  centronics port  : C : centronics port  : C

parallel port on a hercules grapics card 	              : E

IBM serial card as COM1 in PC-slot     : F :                  : F

The program CHNGDEV.EXE changes which device is hooked to which

channel.   It's input is a text file that has three numbers

seperated by commas with no spaces.  The first number is the

channel number.  The second number is the output devive and the

third number is the input device.  For example to change the

console to serial port 1 the line would be as follows.

6,4,5

To change the printer to the centronics port the line would be

as follows.

7,0C,0C

The input file can have more than one line.  CHNGDEV is typically

run from the autoexec.bat file.

example

CHNGDEV CHGFILE.CHG

Here CHGFILE.CHG is a file containing the triplets shown above.

Bios 2.0 also has the ability to change the device driver for any

given channel.  To change the driver for a channel first WRITE AND

DEBUG an assembly language progam to drive your device.  The next

step is to install your driver as part of the bios.  This is done

by making two calls to the function dispatcher ( INT 0F8H IN THE

FILE NEWCALL.ASM ) and then two function calls to the operating

system to exit and stay resident ( INT 21H FUNCTIONS 4DH AND 31H

IN THE REFERENCE MANUAL)

The first call to the function dispatcher is made to channel 0FEH

( file newcall.asm )

ENTRY CONDITION

ES:SI  contains the segment and offset of a buffer big enough to

hold a 4 byte pointer to all external functions.

DX:BX  contains the segment and offset of the ENTRY POINT TO YOUR

DRIVER.  Your driver will be called by the function dispatcher and

must recognise the format it sends.

EXIT

BL  contains the device number assigned to your driver.

The second call to the function dispatcher is made to channel 0FFH

( file newcall.asm )

ENTRY CONDITION

AL  channel number to reassign, example 7 for printer

bl  new input device

bh  new output device

EXIT

bl old input device

bh old output device

The third and fourth calls are made to MS-DOS functions 4DH and

31H chapter 2 reference manual.

These calls exits your program to MS-DOS but saves space for it in

RAM.  You need to know space to how much memory to save for your

program.  This number is simply ((offset first instuction - offset

last instuction +1 ) / 16 ) + 10H.  I have found that the 10H is

not allways enough so if you go to rats try and increase this number

first. Make sure to include your data segment.

SAMPLE PROGRAM DEVPRN.ASM

O L D   M O N I T O R   C A L L S

All but two old monitor calls are supported by the new bios if you

own the pc expansion board.  Cpm can be run if you want although

the old utilites still need to be written for the new bios ( sliform

& setup).  To run Cpm this is what you do.  First you need a Slicer

system with two disk drives and the new bios on it.  Then run the

program LOADER3.EXE. CPM will boot if Slicer CPM.SYS is on the disk.

CONST	HOME		DUHIGH	    DOLMESS	CLRSCR (with pc board )

CONIN	READ		DULOW	    MESSAGE	CRTSET (with pc board )

CONOUT	WRITE		LSTOUT	    HEXWORD

DUBIN	DUON		LSTST	    HEXBYTE

DUBOUT	DUOFF		DSKCHECK    DECDISP

These calls are supported through INT 3BH and INT FBH.  The

reason for having two interrupts that support the same calls is

that dos may change and use INT 3BH.  So if your old programs use

INT 3BH you should move the vector at 0:03EC & 0:03ED to 0:EC &

0:ED to be safe.

U S I N G   A   S E R I A L   T E R M I N E L

The serial terminal runs out of port A on the Slicer board.

The communication protocall for the serial terminal is as

follows.

JB2 PINS ONE & TWO

9600 baud, eight bits per character, no parity, two stop bits.

No parity means no bit is present !

JB2 PINS THREE & FOUR

Autobaud detect after double reset and carrage return, eight

bits per character, no parity, two stop bits.  Console is set to

terminal even if P C board present.

JB2 PINS FOUR & FIVE

Autobaud detect after double reset and carrage return, eight

bits per character, no parity, two stop bits. Console is set to

P C board if present.

ALL SETTINGS REQUIRE HARDWARE HANDSHAKING !

The program devcon.asm is a serial terminal con driver for

MS-DOS.  It is meant to translate ansi escape sequences for cursor

positioning.  It can translate input, output, or both input and

output.  The programs default table translates for the Slicer

video board if the extended tty is on.  The program can be changed

to use your terminal cursor position sequences ether by reassembling

with masm assembler, or by disassembling the program with debug

and tracing through it changing the tables.

The control devcon was intended to provide is as follows.

input control

cursor up, down, right, left, F1, F2, F3

output control

cursor up, down, left, right, home

erase from cursor to end of page

erase from begining of page to cursor

erase from cursor to end of line

erase from begining of line to cursor

erase entire line

begin reverse video

exit reverese video

save cursor position

restore cursor position

direct cursor positioning

If anyone modiffies this program for a specific terminal please send

Slicer a copy so that others can use it.  THANK YOU

P C   E X P A N S I O N   B O A R D    O W N E R S

The PC board is a must for anyone interested in IBM compatibility.

It gives enough compatibility for many more programs to run.  The

text mode on the video is comatible and runs very fast.  Run some

macros for 123 and you will see this is true.  There are however

some changes that can increase the IBM compatibility.

*************************************************************************

H E R C U L E S   M O N O C H R O M E   G R A P H I C S

If you wish to use a Hercules graphics card it can be done.

There are a few changes that need to be made to the PC Expansion

board.  The changes involve making the pc slot work for memory

and I/O in the ranges of the monochome video, and disabling Slicer

monochrome video.  This modification will slow down the text display

to IBM PC speed but will allow programs that use Hercules graphics

to run (provided they use no I/O calls other than video).  There

are three steps in enabling the pc slot for monochrome video.  First

all chips that are used in Slicer video that might cause buss conflicts

need to be removed.  Second the PAL in U9 needs to be changed.  Third

add a jumper to pins five and six of JB2 on the Slicer.  The changes

in detail are as follows.

1.  CHIPS TO BE REMOVED

U22  				6845

U23, U24  			video RAM

U25, U26  			LS374

U27       			PAL 12L6

U15, U17, U28       		LS244

U30  				PAL 12L10

U16, U18			LS373

U34				2764

U19, U20, U21			LS157

U10, U11			video ROM

U57				LS86

U12				LS139

2.  PAL 18L4 U9

PAL18L4

HERC

DECODER

SLICER COMPUTERS INC, MPLS. MN

A16 A15 A11 A10 A9 A8 A7 A6 A5 A4 PALCS GND

S2 PCS6 MCS1 DEN /CS2 /CS1 /MMA /BDEN A19 A18 A17 VCC

/MMA=/DEN*S2*A19*/A18*A17*A16*/A15

/CS1=/DEN*S2*A19*/A18*A17*A16+

/DEN*/S2*/A15*/A11*/A10*A9*A8*A7+

/DEN*/S2*/A15*/A11*/A10*A9*A8*/A7*A6*A5*A4

/BDEN=/DEN*/PCS6+/DEN*/MCS1+

/DEN*S2*A19*/A18*A17*A16+

/DEN*/S2*/A15*/A11*/A10*A9*A8*A7+

/DEN*/S2*/A15*/A11*/A10*A9*A8*/A7*A6*A5*A4

DESCRIPTION

3.	Also on power up use chngdev with the input file 7,0e,0e to use

the parallel port on the hercules card for your printer.  Change the

jumper JB2 ON THE SLICER, jumper pins one and two and four and five.

*************************************************************************

U S I N G   A N   I B M   C O M P A T A B L E

S E R I A L   C A R D

The BIOS supports an IBM compatable serial card installed as

COM1.  The port should then be put into the BIOS as a device with

CHNGDEV.EXE.

Interrupt lines for the PC-slots are not wired.  Therefore for

interrupts to work you first need to wire the interrupt line INT4

on the PC slot to the 80186 interrupt system.  Int3 is my choice

because it leaves interrupt vector 0ch open for programs to use

the as COM interrupt.  This hardware change is only valid for

Slicer boards REV E and E2.

The hardware changes needed are simply wire INT4 of the

PC-slot to pin 35 on J2 of the Slicer bus.  Then cut the trace on

the Slicer board 2681 at pin 36.  Now run a jumper from the 2681

pin 36 to J2 pin 35.  IF YOU USE THE SERIAL CARD THESE JUMPERS

MUST BE ADDED !!!  THIS IS A GOOD CHANGE TO MAKE BECAUSE MANY

COMMUNICATION PROGRAMS WILL RUN !!!

*************************************************************************

This is a list of modifications that move the keyboard from 300h to 60h

which is where IBM's keyboard is. Many programs directly read from IBM's

keyboard port at 60h. This lets some programs steal the keyboard, INT 9,

but you must be able to change the ROM.

NOTE THAT THIS MODIFICATION SHOULD BE DONE BY QUALIFIED TECHNICIANS ONLY.

It involes replacing PAL U9,and cutting some traces and a few jumpers on

both the slicer and video board.

****On the PC Expansion Board make the following changes:

Cut trace the following traces at U9 pin 14

U9 pin 17

U9 pin 19

J2 pin 38 at bus

Jumper J2 pin 37 to U9 pin 14

J2 pin 38 to U9 pin 19

****On the Slicer make this changes:

Cut trace from J3 pin 37

"    "     "  J3 pin 38

"    "     "  U32 pin 3

Jumper U32 pin 3 to J3 pin 38

"    U16 pin 25 to J3 pin 37

****Obtain a new PAL for U9 on the video board from slicer for $15.00

and specify for keyboard move only or keyboard move and hercules modification.

Obtain a new set of roms from Slicer specifing keybord move version.

CALL US BEFORE ORDERING FOR PRICE AND AVAILABLITY

*************************************************************************

OTHER CHANGES TO PC EXPANSION BOARD

There a few other changes that can be made to increase IBM

compatibility.  One change was made to improve the timming of the

read and write signals.  A second change was made because ALE line

to pc slot was inverted.  I don't think this would cause many problems

since addresses and data are already separated at the slot.

1.  CUT TRACE AT U53 16R4 PIN 1

JUMPER U39 LS163 PIN 13 TO U53 16R4 PIN 1

2.  CHANGE LINE IN PAL PROGRAM FOR U53 16R4

IF(VCC)/ALE=FF1*/FF2*/CLK+/FF1*FF2+FF1*FF2+FF3+/FF3*/FF2*/FF1

For your information:  This PAL is probobly the most complicated

on the board.  The outputs of the PAL create the 8088 timming signals

as follows.

T1 = FF1 * /FF2

T2 = /FF1 * FF2

T3 = FF1 * FF2

T4 = FF3

PASS = /FF3 * /FF2 * /FF1

FF4 = Second bus cycle on word transfers

*************************************************************************

PROGRAM STEAL210.ASM

This program does not allow other programs to steal interrupt

13h through DOS.  This is important because this is are timer interrupt.

The program insted returns a vector that is more like the floppy disk

call namely 0fdh.  It also does not allow programs to steal interrupt

25h through DOS.  This gave XTALK access to the floppys without crasing.

For best results with XTALK use a ram disk.

*************************************************************************

CHANGE FROM THE PC OWNERS MANUAL

The two new calls to interrupt 10h were added to turn on and

off the extended tty mode.  A call with ah register equal FB will

send all output through extended tty.  This is then turned off

with a call with ah register equal FC.  Probibly the only time

you would want to turn it on is for ansi sequence translated by

the devcon con driver.  The call documented in the PC  board

manual with ah equal to ff was changed to fd.  This is because

two new calls to the IBM bios were added since that document

was printed they were ah equal ff and ah equal fe ( for topview ).

***********************************************************************

E X P A N S I O N   B O A R D   O W N E R S

************************************************************************

R A M D I S K :

The program RAMDISK.SYS is a memory disk for the slicer.  It will

use the bank switched memory on the megabyte expansion board

making your Slicer awesome.  There are also previsions to disable

the use of bank switched memory if desired.  To install the driver

there should be a line in the CONFIG.SYS file as follows.

DEVICE=RAMDISK.SYS 200 { /NOEXTEND }

The number after ramdisk.sys is the amount of memory in kilobytes

to use for the ramdisk.  The optional /NOEXTEND disables the use

of bank switched memory on the megabyte expansion board.  If the

megabyte board is not present the /NOEXTEND makes no difference.

NOTE: You cannot use ramdisk.sys and emm.bin at the same time.

**************************************************************************

EMM.BIN

Memory Manager using Lotus, Intel, Microsoft expaned

memory standard (LIM).

It may only be used with a 1 MEG memory expansion board.

It cannot be used with RAMDISK.SYS

In your CONFIG.SYS file you must have DEVICE=EMM.BIN for it to work.

The driver assumes a 256K Slicer and a fully populated 1 MEG board.

***************************************************************************

R E A L   T I M E   C L O C K :

The file SETRTC.com sets the real time clock on the expansion board.

The file SETCLK reads the real time clock on the expansion board and

sets the MS-DOS clock.  IT ( SETCLK ) SHOULD BE RUN FROM AUTOEXEC.BAT

BECAUSE IT NEEDS COMMAMD.COM TO WORK

***************************************************************************

P A R A L L E L   P R I N T E R

There are two handshaking setup files supplied PARPRINT.PRN and DEVPRN.EXE

These two programs follow the two ways of changing the driver for

the printer channel discribed in the bios section.  The main difference

between these two is that devprn uses busy and acknowledge while the driver

in the bios only uses the latched acknowledge line.  A note about the busy

line, it is not wired on the 256 board.  To install a busy line on the 256

board run a jumper from J11-21 to the sc2681 pin 36.  This is not

inverted as on the Meg expansion board ( there are no spare

inverters ).  This difference is why the bios uses acknowledge only.

Devprn tests busy for the 256 board and must be changed for the Meg board.

RUN THESE PROGRAMS FROM AUTOEXEC.BAT.  If these programs do not work write

your own driver and install it into the bios a good example is supplied

DEVPRN.ASM.  Try the channel changer and parprint.prn first because it

uses no ram.

If your printer goes into the dubble space mode you may want

to do the following.  Cut the trace at J11 pin 27 that goes to

the gound bus.  A gound on this pin may force your printer into

the automatic line feed mode.  Check YOUR PRINTER DRAWINGS!

***************************************************************************

E X P A N S I O N   S E R I A L   P O R T S

At this time two of the four serial ports on the expansion board

are supported by the bios.  The two ports that are supported are the

two ports off of the 2681 serial chip.  The default setup for

these ports is  1200 baud,eight bits per character, no parity, two

stop bits.  I should mention here that calls to int 14h support the

Slicer serial chips or an IBM compatable serial card if it is installed

in the BIOS with CHNGDEV.  All six serial ports can be set up using the

batch file SETPORT.BAT.  First you edit the file INIT.IN then run

SETPORT.BAT. This will set up the baud rate, parity, and stop bits

for all of the six serial ports available.  It does not have the ability

to change hardware handshaking requirement at this time.  All ports

supported by the BIOS require handshaking except port B on the Slicer

board. If you use the ZILOG chips you will need to steal int 0ch

because these chips are not yet supported.  In your sevice routine

check for a ZILOG interrupt, if this chip did not interrupt pass

it on to the old service routine.

***************************************************************************

F E A T U R E S

1.  All necessary IBM ROM calls work

2.  Capability to boot from hard disk

3.  Capability to run two hard disks

4.  Faster disk operations

5.  Guide for installing IBM serial card (must have PC Video Expansion)

Serial Mouse

Modem

Compatiable with communications programs for IBM PC (example:Xtalk)

6.  May use color video card  (must have PC Video Expansion)

7.  Guide for installing Hercules Graphics Card  (must have PC Video Expansion)

W E   D O   N O T   E X P E C T

1.  We do not expect complete IBM compatability.  We DO NOT use the same

chips so if a program accesses IBM hardware directly ( an I/O call

not through the operating system or bios) it may not run.

Look at the section on the PC expansion board for compatibility

questions.

***************************************************************************

T R O U B L E   S H O O T I N G

These are the things that should be checked first.

1.  Are the EPROMs in correctly ?  Jumper JB1 is set for 27128, ODD in

U6 EVEN in U7 with pin one at the top of the board.  With the top

being the edge at the top of the Slicer logo.

2.	Is JB2 jumper correctly ?  Pins 1 and 2 = 9600 baud or  PC board

is present installed.  Pins 3 and 4 Serial terminal autobaud detect

after two carrage returns on port A.

3.  Check the interrupt jumpers on your boards.  PC board keyboard

is INT2 and expansion board is INT0 on rev E and E2.  Jumpers are

as follows, on the PC board J9 pins 1 and 2 are jumpered and on the

expansion board J5 pins 1 and 2.

4.  WILD INTERRUPT

This is really a feature HA HA.  Vector 6 is the most common, it is

a bad opcode interrupt and can indicate some of the following problems.

If it happens on boot the disk is probobly bad.  It could also indicate

a bad buss cable or disk cable or bad memory.  Other common vectors are

2 or divide by 0.  These interrupts are NMI and DIVIDE BY 0 interrupts.

Since NMI is tide to ground you should check your power and ground.

Divide error is harder to find but could be any of the above problems.

Divide can come through DOS bad opcode ( INT 6  ) or though INT 0.

These are harder to track but can be bad memory.  Wild interrupts

through other vectors are usually program relalted.  The first thing to

try is set the interrupt vector to IRET instruction.  There is one at

the top of ROM for the is purpose, it's address is segment F800 offset

7FFD.  This can be done with debug or the monitor.  A good way to find

the caller is to look at the stack when you are sent to RATS since the

segment offset and flags are pushed here when the interrupt occurs.

If these are not consitenly the same check power ground and memory.

If they are always the same it is probobly a compatibiliy or BIOS

error.

TERMINAL WON'T RUN

1.	Terminal data format 8 characters, no parity, two stop.  This

means no parity bit can be sent.  If you can access the PC MODE

command run it from a autoexec.bat on your Slicer boot disk.

2.	Is your terminal useing CTS and DTR ?  If you boot but can't

get a response from your terminal keyboard try this.  Go to a machine

running DOS on 40 track disks incert your disk and type the following.

copy con:autoexec.bat

debug <handoff

^Z

copy con:handoff

O84,15

O80,13

O80,0F

Q

^Z

Now go and try to reboot.  The handshaking on port A is disabled.

CAN'T GET PC BOARD RUNNING

1.  Check for the following since this is what the ROMS do.  First

they check JB2 ON THE SLICER BOARD.  Then they look for video

controller at I/O address 3B4h by doing the following ( so you

should try this ).  Write 0fh to I/O address 3b4h  then write

55h to I/O address 3b5h.  Now read 3b5h and check for 55h.  Then

write 0aah to 3b5h and check it.  If all this goes well call Slicer

for the address that vector 10h should be pointing to if these are

the same you should be able to initialize video through calls to

INT 10H and then use CHNGDEV as described in the section on the

BIOS.

2.  Check that you can write to U48 at I/O address 300h.  This must

be done with a scope.  Check for activity on the chip select and

data lines.  This I/O port selects character mode or graphics mode,

resets the keyboard and sets the segment of the I/O slot.  Check

these outputs.  One thing if you can't write this and have an expansion

board, check the expansion board J4 it should be jummpered to use

PCS4 since the keyboard uses PCS6.

3.	If you have keyboard problems check the above and also look at

U47 on READS from I/O address 300h.  After a reset the data input

from the keyboard should read 0aah.

4.	If you CAN read and write the above I/O BUMMER.  It could be

a million different things PALS MEMORY OR LATCHES.  The check

sum on the PALS can be checked by calling Slicer but you must

have access to a PAL programer.  The ROMS on the PC board are not

used, so it's not that.  Check the board for shorts and hot chips.

These seem to be the main problems people have in getting

started.  I hope they have helped you.  When you have a problem

alway disconect as many boards and devices as possible in order

to isolate the problem.  Call Slicer for help.  It helps me if

you can be at your computer when you call.  Scopes are nice and

use of DEBUG AND OR THE MONITOR A MUST in debugging. Have your

manuals also for chip and pin refferances.  Ask for John,  Brad

or Jonathan  Thank you for reading this stuff!!

Slicer computers