\JC

INSTALLING THE X120 BOARD ON A BIG BOARD I

\JL

The Emerald Microware X120 Double Density adaptor board may be

used on the Ferguson Big Board I, as well as the Xerox 820-1,

since they are functionally the same board.  However, some of the

parts are not the same, and in any case have different locations

on the board.  The installation is slightly less elegant, in that

though J1 can still plug into the WD1771 location, but a single

ended dip jumper is necessary to pick up and return the signals

that are handled through J2.  The crystal for the baud rate

generator will also need to be laid down, as with other upgrades.

You can ignore the composite video adaptor on the X120 board,

since the BBI already has this feature.  The SYSPIO which is U105

on the Xerox board is U111 on the BBI, and the WD1771 is U102. The

7445 at U109 is the only component that needs to be removed from

the BBI, and no cuts should be necessary. I do recommend that the

1771 socket be replaced with female headers for best installation.

The signal list for J2 on the X120 board is as follows:

J2 pin       Big Board I, chip-pin

1        U111-13

2        NOT USED, CAN BE RUN TO BBI J1-4

3        U111-15

4        U109-1

5        VIDEO VSYNC, NOT NECESSARY

6        VIDEO VSYNC RETURN, NOT NECESSARY

7        GND, 0 VOLTS

8        U111-8

9        U111-9

10        VIDEO HSYNC RETURN, NOT NECESSARY

11        VIDEO HSYNC, NOT NECESSARY

12        VIDOE RETURN, NOT NECESSARY

13        VIDEO, NOT NECESSARY

14        U109, 2

15        U111,14

16        +5 VOLTS DC

Only the signals connecting to U111 are necessary if you do not

plan to use J1 to connect 8" drives on the BBI itself.   If J1 on

the BBI is to still be used to connect drives, pin 4 of U104

should be pulled out of its socket, and connected to pin 14 of

U104, and a jumper is required on the X120 board between J1-28 and

the 2791 pin 28 to return the HEAD LOAD signal to the BBI.  The

DVSEL2 and DVSEL3 signals are not returned to the BBI, but may be

picked up from pins 5 (DVSEL2) and 9 (DVSEL3) of the 7445 on the

X120 board if they are needed.  The BBI will now have true side

select capabilities also.